How you cut and make up the bags will depend if you are using an overlocker or a regular sewing machine. The cutting guide is really just a guide as the bags do not need to be a specific size.  It is more about re-using old materials to create a useable bag to cut plastic waste.

Cutting guide sewing:

Main bag - 1 x 18”x 37” (or 2 x 18” x 19” if you are using a directional print)

Handles – 2 x 18” x 5”

Cutting guide overlocker:

Main bag – 1 x 17” x 37” (or 2 x 17” x 18.5” if you are using a directional print)

Handles – 2 x 18” x 5”

Instructions

Handles:

- Press lengthways WST, the press in each raw edge to the crease line.  Finish by pressing both folded edges on top of each other to enclose all raw edges.

- Top stitch down both sides of the strap with a 1/8” allowance

Main Bag (If overlocking)

Place bag outer RST and sew up both sides and (if required) along the bottom edge then continue as per instructions below:

Main Bag (all methods)

- Fold over the top edge of the bag to the WS by ½” and then again by 1”

- Centrally position the handles being careful not to twist them (approx. 6” in from each side) and clip or pin the ends in place underneath the fold with the handle hanging down towards the bottom of the bag

- Sew each handle in place by sewing around the bottom edge of the hem with a 1/8” allowance

- Pull the handles up to the top of the bag and sew again around the top edge of the bag with a 1/8” allowance securing the handles up and in place

- Sew on your MORSBAGs label

If you have made your bag on an overlocker it is finished at this point.

If you are using a sewing machine then please continue with the steps below:

- (We will be sewing French seams) Sew side seams WST with a 1/8” allowance.  Trim any loose threads away.

- Turn bag RST and press well along both sewn edges to get a crisp seam.

- Sew down both sides again with a ¼” allowance to enclose the raw edges.

TaDa!!! Turn your bag in the right way and it is now complete.

Be sure to tell us you’ve made a bag so we can log it on our POD as a BTHQ bag.